108 Liberty St | Metuchen NJ 08840
800-422-1751 (t) | 732-452-0444 (f)
www.ushalonbank.com

GREAT LAKES CHEMICAL -- HALON 1211 - EXTINGUISHER,FIRE,VAPORIZING LIQUID

MATERIAL SAFETY DATA SHEET

NSN: 4210011009085

Manufacturer's CAGE: 2E394

Part No. Indicator: A

Part Number/Trade Name: HALON 1211

===========================================================================

General Information

===========================================================================

Item Name: EXTINGUISHER,FIRE,VAPORIZING LIQUID

Company's Name: GREAT LAKES CHEMICAL CORPORATION

Company's Street: HIGHWAY 52 NW

Company's P. O. Box: 2200

Company's City: WEST LAFAYETTE

Company's State: IN

Company's Country: US

Company's Zip Code: 47906

Company's Emerg Ph #: 501-862-5141 OR 317-463-2511

Company's Info Ph #: 317-497-6100

Record No. For Safety Entry: 002

Tot Safety Entries This Stk#: 003

Status: SE

Date MSDS Prepared: 01SEP90

Safety Data Review Date: 29JUN92

Supply Item Manager: 75

MSDS Serial Number: BHQBQ

Specification Number: A-A-1108B

Spec Type, Grade, Class: TY A

Hazard Characteristic Code: G3

Unit Of Issue: EA

Unit Of Issue Container Qty: 3.0 LBS

Type Of Container: CYLINDER

Net Unit Weight: 3.0 LBS

===========================================================================

Ingredients/Identity Information

===========================================================================

Proprietary: NO

Ingredient: CHLORODIFLUOROBROMOMETHANE

Ingredient Sequence Number: 01

NIOSH (RTECS) Number: PA5270000

CAS Number: 353-59-3

OSHA PEL: NOT ESTABLISHED

ACGIH TLV: NOT ESTABLISHED

Other Recommended Limit: NONE ESTABLISHED

===========================================================================

Physical/Chemical Characteristics

===========================================================================

Appearance And Odor: COLORLESS GAS AND LIQUID WITH SWEET ODOR.

Boiling Point: 24.8F,-4.0C

Vapor Pressure (MM Hg/70 F): 1897

Vapor Density (Air=1): 5.8

Specific Gravity: 1.83

Decomposition Temperature: UNKNOWN

Solubility In Water: INSOLUBLE

Corrosion Rate (IPY): UNKNOWN

===========================================================================

Fire and Explosion Hazard Data

===========================================================================

Flash Point: NON-FLAMMABLE

Extinguishing Media: HALON 1211 IS A FIRE EXTINGUISHING AGENT. NON-

FLAMMABLE GAS. USE WATER SPRAY TO COOL CYLINDERS INVOLVED IN FIRES.

Special Fire Fighting Proc: WEAR FULL PROTECTIVE CLOTHING AND NIOSH-

APPROVED SELF-CONTAINED BREATHING APPARATUS WITH FULL FACEPIECE OPERATED IN

THE POSITIVE PRESSURE MODE.

Unusual Fire And Expl Hazrds: LOW LEVELS OF TOXIC AND CORROSIVE HALOGENS

OR HALOGEN ACIDS ARE RELEASED IN FIRE SITUATIONS.

===========================================================================

Reactivity Data

===========================================================================

Stability: YES

Cond To Avoid (Stability): HIGH TEMPERATURES

Materials To Avoid: ACTIVE METALS, FIRES OF METAL HYDRIDES, MATERIAL

CONTAINING OWN OXYGEN.

Hazardous Decomp Products: HYDROGEN HALIDES AND HALOGENS AT FLAME

TEMPERATURES IN TRACE AMOUNTS. SMOKING CAN CAUSE FORMATION OF THESES

CHEMICALS.

Hazardous Poly Occur: NO

Conditions To Avoid (Poly): NONE

===========================================================================

Health Hazard Data

===========================================================================

LD50-LC50 Mixture: LC50 (RAT, 4 HR) IS 131,600 PPM

Route Of Entry - Inhalation: YES

Route Of Entry - Skin: NO

Route Of Entry - Ingestion: NO

Health Haz Acute And Chronic: THE UNDERWRITERS LABORATORIES CLASSIFICATION

OF COMPARATIVE LIFE HAZARD OF HALON 1211 IS GROUP 5. AT CONCENTRATIONS

ABOVE 2 PERCENT, HALON 1211 CAN PRODUCE DIZZINESS, IMPAIRED COORDINATION,

AND CARDIAC EFFECTS.

Carcinogenicity - NTP: NO

Carcinogenicity - IARC: NO

Carcinogenicity - OSHA: NO

Signs/Symptoms Of Overexp: EXPOSURE FOR MORE THAN A FEW MINUTES, ABOVE 2

PERCENT (8.5 LB/1000 CU. FT. AIR) CAN PRODUCE DIZZINESS, IMPAIRED

COORDINATION AND CARDIAC EFFECTS.

Med Cond Aggravated By Exp: CARDIAC CONDITIONS MAY BE AGGRAVATED BY

OVEREXPOSURE.

Emergency/First Aid Proc: GET MEDICAL ATTENTION.EYE/SKIN:IMMEDIATELY FLUSH

WITH WATER FOR AT LEAST 15 MINUTES.HOLD EYELIDS OPEN.INHALED:REMOVE TO

FRESH AIR & PROVIDE OXYGEN/CPR IF NEEDED.NOTE:EPINEPHRINE IS

CONTRAINDICATED IN THE TREATMENT OF OVEREXPOSURE TO THIS PRODUCT.

===========================================================================

Precautions for Safe Handling and Use

===========================================================================

Steps If Matl Released/Spill: EVACUATE AREA. WEAR SELF-CONTAINED BREATHING

APPARATUS. VENTILATE ENCLOSED AREAS IN CASE OF DISCHARGE OF HALON 1211.

Waste Disposal Method: RETURN CYLINDERS TO GREAT LAKES CHEMICAL

CORPORATION, EL DORADO, ARKANSAS.

Precautions-Handling/Storing: STORAGE-STORE TEMPERATURE SHOULD NOT EXCEED

125F.PROTECT CYLINDERS FROM PHYSICAL DAMAGE.

Other Precautions: CYLINDERS SHOULD BE SECURED FOR STORAGE WITH VALVE

PROTECTION CAP IN PLACE. AVOID PROLONGED EXPOSURE TO EXCESSIVE

CONCENTRATIONS.

===========================================================================

Control Measures

===========================================================================

Respiratory Protection: USE SELF-CONTAINED BREATHING APPARATUS IF EXPOSED

TO CONCENTRATIONS GREATER THAN 2 PERCENT.

Ventilation: GOOD GENERAL VENTILATION IS SUFFICIENT FOR MOST CONDITIONS.

Protective Gloves: LINED NEOPRENE/LEATHER AGAINST FROSTBITE

Eye Protection: CHEMICAL SAFETY GOGGLES

Other Protective Equipment: NONE.

Work Hygienic Practices: DO NOT WEAR CONTACT LENSES.

===========================================================================

Transportation Data

===========================================================================

Trans Data Review Date:   92181

DOT PSN Code: GHB

DOT Proper Shipping Name: FIRE EXTINGUISHERS

DOT Class: 2.2

DOT ID Number: UN1044

DOT Label: NONFLAMMABLE GAS

IMO PSN Code: HGH

IMO Proper Shipping Name: FIRE EXTINGUISHERS

IMO Regulations Page Number: 2141-1

IMO UN Number: 1044

IMO UN Class: 2(2.2)

IMO Subsidiary Risk Label: -

IATA PSN Code: LXU

IATA UN ID Number: 1044

IATA Proper Shipping Name: FIRE EXTINGUISHERS

IATA UN Class: 2.2

IATA Label: NON-FLAMMABLE GAS

AFI PSN Code: LXU

AFI Prop. Shipping Name: FIRE EXTINGUISHERS

AFI Class: 2.2

AFI ID Number: UN1044

AFI Basic Pac Ref: 6-6,6-12

N.O.S. Shipping Name: CONTAINS BROMOCHLORODIFLUOROMETHANE

Additional Trans Data: MANUFACTURER STATES THAT THE PRODUCT IS NOT

REGULATED FOR (DOT) DOMESTIC SHIPMENT AND SHIPPING NAME FOR EXPORT IS

CHLORODIFULOROBROMOMETHANE, NONFLAMMABLE GAS, UN 1974.

===========================================================================

Disposal Data

===========================================================================

===========================================================================

Label Data

===========================================================================

Label Required: YES

Technical Review Date: 29JUN92

MFR Label Number: UNKNOWN

Label Status: F

Common Name: HALON 1211

Signal Word: CAUTION!

Acute Health Hazard-Slight: X

Contact Hazard-Slight: X

Fire Hazard-None: X

Reactivity Hazard-None: X

Special Hazard Precautions: THE UNDERWRITERS LABORATORIES CLASSIFICATION

OF COMPARATIVE LIFE HAZARD OF HALON 1211 IS GROUP 5. AT CONCENTRATIONS

ABOVE 2 PERCENT, HALON 1211 CAN PRODUCE DIZZINESS, IMPAIRED COORDINATION,

AND CARDIAC EFFECTS. STORAGE-STORE TEMPERATURE SHOULD NOT EXCEED 125F.

PROTECT CYLINDERS FROM PHYSICAL DAMAGE. FIRST AID- GET MEDICAL ATTENTION.

EYE/SKIN:IMMEDIATELY FLUSH WITH WATER FOR AT LEAST 15 MINUTES.HOLD EYELIDS

EPINEPHRINE IS CONTRAINDICATED IN THE TREATMENT OF OVEREXPOSURE TO THIS

PRODUCT.

Protect Eye: Y

Protect Skin: Y

Protect Respiratory: Y

Label Name: GREAT LAKES CHEMICAL CORPORATION

Label Street: HIGHWAY 52 NW

Label P.O. Box: 2200

Label City: WEST LAFAYETTE

Label State: IN

Label Zip Code: 47906

Label Country: US

Label Emergency Number: 501-862-5141 OR 317-463-2511