108 Liberty St | Metuchen NJ 08840
800-422-1751 (t) | 732-452-0444 (f)
www.ushalonbank.com

GREAT LAKES CHEMICAL –

MONOBROMOTRIFLUOROMETHANE,TECHNICAL

MATERIAL SAFETY DATA SHEET

NSN: 6830009659647

Manufacturer's CAGE: 2E394

Part No. Indicator: A

=====================================================================

General Information

=====================================================================

Item Name: MONOBROMOTRIFLUOROMETHANE,TECHNICAL

Company's Name: GREAT LAKES CHEMICAL CORP

Company's Street: HIGHWAY 52 NW

Company's P. O. Box: 2200

Company's City: WEST LAFAYETTE

Company's State: IN

Company's Country: US

Company's Zip Code: 47906

Company's Emerg Ph #: 501-862-5141

Company's Info Ph #: 317-497-6100

Distributor/Vendor # 1: AMERICAN FIRE AND ELECTRIC INDUSTRIES IN

Distributor/Vendor # 1 Cage: 47229

Record No. For Safety Entry: 003

Tot Safety Entries This Stk#: 007

Status: SE

Date MSDS Prepared: 01JUN93

Safety Data Review Date: 03FEB94

Supply Item Manager: CX

MSDS Serial Number: BHVWL

Specification Number: MIL-M-12218

Hazard Characteristic Code: G3

Unit Of Issue: LB

Unit Of Issue Container Qty: 1-CYLINDER

Type Of Container: CYLINDER

Net Unit Weight: UNKNOWN

=====================================================================

Ingredients/Identity Information

=====================================================================

Proprietary: NO

Ingredient: BROMOTRIFLUOROMETHANE  (SARA III)

Ingredient Sequence Number: 01

Percent: >99.6%

NIOSH (RTECS) Number: PA5425000

CAS Number: 75-63-8

OSHA PEL: 1000 PPM

ACGIH TLV: 1000 PPM; 9394

Other Recommended Limit: NONE SPECIFIED

=====================================================================

Physical/Chemical Characteristics

=====================================================================

Appearance And Odor: COLORLESS GAS. ODORLESS.

Boiling Point: -72F,-58C

Melting Point: -270F,-168C

Vapor Pressure (MM Hg/70 F): 6200 @ 0 C

Vapor Density (Air=1): 5.1

Specific Gravity: 1.57

Decomposition Temperature: UNKNOWN

Evaporation Rate And Ref: UNKNOWN

Solubility In Water: NEGLIGIBLE

Percent Volatiles By Volume: GAS

Corrosion Rate (IPY): UNKNOWN

=====================================================================

Fire and Explosion Hazard Data

=====================================================================

Flash Point: NON-FLAMMABLE

Extinguishing Media: A FIRE EXTINGUISHING AGENT. USE MEDIA

SUITABLE FOR FLAMMABLE MATERIALS IN THE AREA.

Special Fire Fighting Proc: USE WATER SPRAY TO COOL CYLINDERS INVOLVED IN

FIRES. WEAR SELF-CONTAINED BREATHING APPARATUS.

Unusual Fire And Expl Hazrds: WHEN EXPOSED TO TEMPERATURES ABOVE 1000F,

HALON 1310 MAY RELEASE TOXIC FUMES OF HYDROGEN FLUORIDE, HYDROGEN BROMIDE,

CARBONYL FLUORIDE AND CARBONYL BROMIDE.

=====================================================================

Reactivity Data

=====================================================================

Stability: YES

Cond To Avoid (Stability): PRODUCT MAY DEGRADE CERTAIN POLYMERIC MATERIALS.

Materials To Avoid: ACTIVE METALS, METAL HYDRIDE FIRES, MATERIAL

CONTAINING OWN OXYGEN.PRODUCT WILL NOT EXTINGUISH FIRES OF THESE MATERIALS.

Hazardous Decomp Products: IN PRESENCE OF FLAMES/HOT SURFACES: HYDROGEN

FLUORIDE, HYDROGEN BROMIDE, BROMINE, CARBONYL FLUORIDE, CARBONYL BROMIDE.

Hazardous Poly Occur: NO

Conditions To Avoid (Poly): WILL NOT OCCUR.

=====================================================================

Health Hazard Data

=====================================================================

LD50-LC50 Mixture: ORAL LD50 (RAT) IS UNKNOWN.

Route Of Entry - Inhalation: YES

Route Of Entry - Skin: NO

Route Of Entry - Ingestion: NO

Health Haz Acute And Chronic: HAS ANESTHETIC AND CARDIAC

EFFECTS IN HUMANS AT CONCENTRATIONS >10% BY VOLUME, CONCENTRATIONS BELOW

10% ARE CONSIDERED SAFE FOR 0 TO 5 MINUTE EXPOSURES. HALON 1301 IS

CLASSIFIED AS GROUP 6 (LEAST TOXIC) IN THE UNDERWRITERS LABORATORIES

CLASSIFICATION OF COMPARATIVE LIFE HAZARDS OF FIRE EXTINGUISHING AGENTS.

Carcinogenicity - NTP: NO

Carcinogenicity - IARC: NO

Carcinogenicity - OSHA: NO

Explanation Carcinogenicity: THIS COMPOUND IS REPORTED TO BE NON-

MUTAGENIC IN THE AMES TEST.

Signs/Symptoms Of Overexp: THE ANESTHETIC EFFECT AT LEVELS

BELOW 10% IS SIMILAR TO A FEELING OF MILD INTOXICATION. ANESTHETIC EFFECTS

END RAPIDLY AFTER EXPOSURE IS TERMINATED. PROLONGED EXPOSURE TO

CONCENTRATIONS ABOVE 10% (15-20%), MAY LEAD TO UNCONSCIOUSNESS AND POSSIBLY

DEATH.

Med Cond Aggravated By Exp: CARDIAC PROBLEMS MAY BE AGGRAVATED BY

OVEREXPOSURE.

Emergency/First Aid Proc: EYES: FLUSH WITH LARGE AMOUNTS OF WATER. GET

MEDICAL ATTENTION. SKIN: FLUSH AREA WITH WATER. IF FROSTBITE OCCURS, GET

MEDICAL ATTENTION. INHALATION: MOVE TO FRESH AIR. IF NOT BREATHING, GIVE

NOT CONSIDERED A LIKELY SOURCE OF EXPOSURE. HOWEVER, IF INGESTED, GET

MEDICAL ATTENTION.

=====================================================================

Precautions for Safe Handling and Use

=====================================================================

Steps If Matl Released/Spill: EVACUATE AREA. WEAR SELF-CONTAINED BREATHING

APPARATUS. VENTILATE ENCLOSED AREAS.

Neutralizing Agent: NONE SPECIFIED BY MANUFACTURER.

Waste Disposal Method: DISPOSE OF WASTE IN ACCORDANCE WITH LOCAL, STATE

AND FEDERAL REGULATIONS. RETURN CYLINDERS TO GREAT LAKES CHEMICAL

CORPORATION, EL DORADO, ARKANSAS.

Precautions-Handling/Storing: PROTECT CYLINDERS FROM DAMAGE. STORAGE TEMP.

SHOULD NOT EXCEED 125F. CYLINDERS SHOULD BE SECURED FOR STORAGE WITH VALVE

PROTECTION CAP INSTALLED.

Other Precautions: DETERMINE THAT PIPING IS EMPTY BEFORE DOING MAINTENANCE

WORK. PERIODICALLY CHECK CYLINDERS TO ENSURE VALVES ARE NOT LEAKING. DO NOT

WEAR CONTACT LENSES.

=====================================================================

Control Measures

=====================================================================

Respiratory Protection: WEAR NIOSH APPROVED SELF-CONTAINED BREATHING

APPARATUS AT CONCENTRATIONS GREATER THAN 10,000 PPM AS RECOMMENDED BY

NIOSH. CONSULT ANSI STANDARD AND OSHA REGULATIONS CONCERNING THE USE OF

RESPIRATORY EQUIPMENT.

Ventilation: MECHANICAL: ALL INDOOR AREAS SHOULD BE WELL VENTILATED. USE

LOCAL EXHAUST FOR MATERIAL TRANSFER AND HANDLING OPERATIONS.

Protective Gloves: LINED NEOPRENE OR LEATHER RECOMMENDED.

Eye Protection: CHEMICAL SAFETY GOGGLES.

Other Protective Equipment: DO NOT WEAR CONTACT LENSES.

Work Hygienic Practices: WASH AFTER HANDLING AND BEFORE EATING, DRINKING,

OR SMOKING.

Suppl. Safety & Health Data: AVOID EXPOSURE TO THIS PRODUCT AND

DECOMPOSITION PRODUCTS WHEN WEARING CONTACT LENSES; CLOUDING OF LENSES MAY OCCUR. SMOKING IN AREAS WHERE THIS CHEMICAL IS RELEASED CAN CAUSE FORMATIONOF TOXIC AND IRRITATING HALOGENS AND HALOGEN ACIDS WHICH COULD BE INHALED WITH THE SMOKE.

=====================================================================

Transportation Data

=====================================================================

Trans Data Review Date:   94034

DOT PSN Code: CEX

DOT Proper Shipping Name: BROMOTRIFLUOROMETHANE OR REFRIGERANT GAS, R

13B1.

DOT Class: 2.2

DOT ID Number: UN1009

DOT Label: NONFLAMMABLE GASS

IMO PSN Code: CMN

IMO Proper Shipping Name: BROMOTRIFLUOROMETHANE

IMO Regulations Page Number: 2109

IMO UN Number: 1009

IMO UN Class: 2(2.2)

IMO Subsidiary Risk Label: -

IATA PSN Code: DYC

IATA UN ID Number: 1009

IATA Proper Shipping Name: BROMOTRIFLUOROMETHANE

IATA UN Class: 2.2

IATA Label: NON-FLAMMABLE GAS

AFI PSN Code: DYC

AFI Prop. Shipping Name: BROMOTRIFLUOROMETHANE (R13B1)

AFI Class: 2.2

AFI ID Number: UN1009

AFI Basic Pac Ref: 6-6,6-8

Additional Trans Data:

=====================================================================

Disposal Data

=====================================================================

=====================================================================

Label Data

=====================================================================

Label Required: YES

Technical Review Date: 03FEB94

Label Status: F

Chronic Hazard: YES

Signal Word: DANGER!

Acute Health Hazard-Severe: X

Contact Hazard-Moderate: X

Fire Hazard-None: X

Reactivity Hazard-None: X

Special Hazard Precautions: HAS ANESTHETIC AND CARDIAC EFFECTS

IN HUMANS AT CONCENTRATIONS >10% BY VOLUME, CONCENTRATIONS BELOW 10% ARE CONSIDERED SAFE FOR 0 TO 5 MINUTE EXPOSURES. IS CLASSIFIED AS

GROUP 6 (LEAST TOXIC) IN THE UNDERWRITERS LABORATORIES CLASSIFICATION OF

FLUSH WITH LARGE AMOUNTS OF WATER. GET MEDICAL ATTENTION. SKIN: FLUSH AREA WITH WATER. IF FROSTBITE OCCURS, GET MEDICAL ATTENTION. INHALATION: MOVE TO FRESH AIR. IF NOT BREATHING, GIVE CPR. IF BREATHING DIFFICULT, GIVE OXYGEN. GET MEDICAL ATTENTION. INGESTION: NOT LIKELY TO OCCUR. HOWEVER, GET MEDICAL

ATTENTION.

Protect Eye: Y

Protect Skin: Y

Protect Respiratory: Y

Label Name: GREAT LAKES CHEMICAL CORP

Label Street: HIGHWAY 52 NW

Label P.O. Box: 2200

Label City: WEST LAFAYETTE

Label State: IN

Label Zip Code: 47906

Label Country: US

Label Emergency Number: 501-862-5141 OR 317-497-6100